Moen Incorporated	January 2010

25300 Al Moen Drive

North Olmsted, Ohio  44070

Toll Free	(800) 289-6636

Website	www.moencommercial.com

E-mail	specmoen@moen.com

Moen Canada:

2816 Bristol Circle

Oakville, Ontario

L6H 5S7

Toll Free	(800) 465-6130

Website	www.moen.ca

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice, Fifth Edition.

This section must be carefully reviewed and edited by the Architect to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section numbers and titles are from MasterFormat 1995 Edition, with section numbers and titles from MasterFormat 2004 Edition in parentheses.  Delete version not required.

SECTION 15410 (22 42 39)

COMMERCIAL FAUCETS

Specifier Notes:  This section covers Moen Commercial Model 8425 single-handle lavatory faucet with metal grid strainer waste assembly.  Consult Moen Commercial for assistance in editing this section for the specific application.

PART 1	GENERAL

- 1.1	SECTION INCLUDES

A.	Single-handle lavatory faucets with metal grid strainer waste assembly waste.

- 1.2	RELATED REQUIREMENTS

Specifier Notes:  Edit the following list of related sections as required.  Delete related sections not required.  List other sections with work directly related to this section.

A.	Section 15140 (22 11 00) – Domestic Water Piping (Facility Water Distribution):  Supply connections to commercial plumbing fixtures.

B.	Section 15150 (22 13 00) – Sanitary Waste and Vent Piping (Facility Sanitary Sewerage):  Waste connections to commercial plumbing fixtures.

- 1.3	REFERENCES

Specifier Notes:  List references mentioned in this section, complete with designations and titles.  Delete references not included in this edited section.  This article does not require compliance with references, but is merely a listing of those used.

A.	ASME A112.18.1/CSA B125.1 – Plumbing Supply Fittings.

B.	ANSI/NSF 61/9 – Drinking Water System Components – Health Effects.

C.	California Proposition 65.

D.	Federal Safe Drinking Water Act.

E.	ADA – Americans with Disabilities Act.

- 1.4	SUBMITTALS

- Specifier Notes:  Edit submittal requirements as required.  Delete submittals not required.

A.	Comply with Section 01330 (01 33 00) – Submittal Procedures.

B.	Product Data:  Submit manufacturer’s product data, including installation instructions.

- Specifier Notes:  Delete samples if not required.

C.	Samples:  Submit manufacturer’s samples for selection of finish.

D.	Manufacturer’s Certification:  Submit manufacturer’s certification that materials comply with specified requirements and are suitable for intended application.

E.	Operation and Maintenance Instructions:  Submit manufacturer’s operation and maintenance instructions, including cleaning instructions and parts list.

F.	Warranty Documentation:  Submit manufacturer’s standard warranty.

- 1.5	QUALITY ASSURANCE

A.	Manufacturer’s Qualifications:  Manufacturer regularly engaged for past 10 years in manufacture of commercial plumbing fixtures of similar type to that specified.

- 1.6	DELIVERY, STORAGE, AND HANDLING

A.	Delivery and Acceptance Requirements:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage and Handling Requirements:

1.	Store and handle materials in accordance with manufacturer’s instructions.

2.	Keep materials in manufacturer’s original, unopened containers and packaging until installation.

3.	Store materials in clean, dry area indoors.

4.	Protect materials and finish during storage, handling, and installation to prevent damage.

- 1.7	WARRANTY

A.	Warranty Period:  5 years.

PART 2	PRODUCTS

- 2.1	MANUFACTURER

A.	Moen Incorporated, 25300 Al Moen Drive, North Olmsted, Ohio 44070.  Toll Free (800) 289-6636.  Website www.moencommercial.com.  E-mail specmoen@moen.com.

B.	Moen Canada: 2816 Bristol Circle, Oakville, Ontario, L6H 5S7. Toll Free (800) 465-6130.

Website www.moen.ca.

- 2.2	COMMERCIAL FAUCETS

- Specifier Notes:  Refer to Moen Commercial Products Catalog for additional aerators.

A.	Single-Handle Lavatory Faucets with Metal Grid Strainer Waste Assembly:  Model 8425.

1.	Metal construction with chrome-plated finish.

2.      All metal grid strainer drain assembly

3.	4-inch centerset.

4.	1/2-inch IPS adapters.

5.	Vandal-resistant screws.

6.	Operation:

a.	4-inch lever-style handle with hot and cold color indicators.

b.	Pull-on/push-off.

c.	Temperature controlled by 180-degree arc of handle travel.

7.	Flow:  Aerator limited to maximum 2.2 gpm (8.3 L/min).

8.	Cartridge:

a.	1225 washerless cartridge design.

b.	Nonmetallic/nonferrous and stainless steel material.

9.	Compliance:

a.	ASME A112.18.1/CSA B125.1.

b.	ANSI/NSF 61/9.

c.	California Proposition 65.

d.	Federal Safe Drinking Water Act.

e.	Handles: ADA.

PART 3	EXECUTION

- 3.1	EXAMINATION

A.	Examine areas to receive single-handle lavatory faucets.

B.	Notify Architect of conditions that would adversely affect installation or subsequent use.

C.	Do not begin installation until unacceptable conditions are corrected.

- 3.2	INSTALLATION

A.	Install single-handle lavatory faucets in accordance with manufacturer’s instructions at locations indicated on the Drawings.

B.	Install single-handle lavatory faucets level, plumb, and without leaks.

- 3.3	ADJUSTING

A.	Adjust single-handle lavatory faucets for proper operation in accordance with manufacturer’s instructions.

B.	Remove and replace damaged components that cannot be successfully repaired.

- 3.4	CLEANING

A.	Clean single-handle lavatory faucets promptly after installation in accordance with manufacturer’s instructions.

B.	Do not use harsh cleaning materials or methods that could damage finish.

- 3.5	PROTECTION

A.	Protect installed single-handle lavatory faucets from damage during construction.

B.	Do not permit use of single-handle lavatory faucets before final acceptance.

END OF SECTION